Introduction

The processor broadly consists of a Datapath and a Control Unit.

Datapath: refers to all the elements that work with data and process them.

Control Unit: generates control signals to direct the processor operation under different situations.

Difference between single clock cycle and pipelined execution:

All instructions do not take the same time to execute. Hence for single clock cycle execution, we need to ensure that the clock period is long enough to accommodate the execution of the slowest instruction.
To speed up the execution process, the Pipelining technique is commonly used where multiple instructions are overlapped in execution. Pipelining however, is prone to different 'hazards' and must deal it.

Here, we have considered single clock cycle execution.

Components of processor datapath:

- Instruction Memory:

- To store the instructions of a program.

- Given the address, it must supply the instruction located at that address.

- We can load instruction memory using readmemh function from a mem file.

module Instruction_Memory(instrn_address, instrn);

input [31:0] instrn_address; //5-bit address holds 8 instructions of 32-bit width

output wire [31:0] instrn;

reg [7:0] instrn_mem [31:0];

initial begin

$readmemh("instrn_memory.mem", instrn_mem);	//load initial values

end

assign instrn = {instrn_mem[instrn_address+3],instrn_mem[instrn_address+2],

instrn_mem[instrn_address+1],instrn_mem[instrn_address]};

endmodule

- Instruction Memory (instrn_memory.mem)

Consider the following assembly code that you want to simulate using this processor.
(The number on the left represents instruction address)

00: add $t1, $t2, $t3        
04: lw $t1, $t2, 16'd4
08: beq $t1, $t2, offset
0C: add $t1, $t2, $t3
10: or $t2, $t3, $t4
14: sw $t1, $t2, offset

Totally we have 6 instructions. Now let us write them in the number code format based on the type of instruction (R or I-Type)

The code for each MIPS instruction is taken from the MIPS instruction manual.

The code now becomes:

00: 6'd0,5'd9,5'd10,5'd11,5'd0,6'h20
04: 6'h23,5'd9,5'd10,16'd4
08: 6'h04,5'd9,5'd9,16'd1 
0C: 6'd0,5'd9,5'd10,5'd11,5'd0,6'h20
10: 6'd0,5'd10,5'd11,5'd12,5'd0,6'h25
14: 6'h2B,5'd9,5'd10,16'd4

Convert this to hexadecimal representation:

00: 01 2A 58 20
04: 8D 2A 00 04
08: 11 29 00 01
0C: 01 2A 58 20
10: 01 4B 60 25
14: AD 2A 00 04

This is the data that we need to load in the instruction memory in a byte-wise manner.

- Program Counter (PC):

- Holds the address of the current instruction.

- For a 32-bit (4-byte) processor, we must increment the address by 4 to fetch the next instruction (as the width of each instruction is 4 bytes). This adder module will be connected to the PC.

- This address increment must happen at every clock cycle and hence, it will be a D Flip-Flop.

- A new instruction is executed every clock cycle.

Verilog Code:

module Program_Counter(clk, rst_n, in_address, out_address);

input clk, rst_n;

input [31:0] in_address;

output reg [31:0] out_address;

always @ (posedge clk or negedge rst_n)

begin

if(!rst_n)

out_address <= 32'd0;

else

out_address <= in_address;

end

- Register File:

- This module will enclose all the independent registers of the processor, to perform write and read operations. MIPS consists of 32 inbuilt registers.

- R-Format instructions have three operands. So, we will need to read 2 dwords(1dword = 4Bytes) from the register file (2 output read ports) and 1 write port (input port) along with a write enable signal that indicates when the data has to be written.

- Example Instruction: add $t1, $t2, $t3
To execute this instruction, we need to read two registers t1 and t2. Add them. Then write the result to register t3.

- In the Verilog code below, combinational read is done from the register memory using assign statement. But usually, read data will appear only after 1 clock cycle (flopped)

- We can load register memory using readmemh function from a mem file.

Verilog Code:

module Register_File(clk, rst_n, read_addr1, read_addr2, write_en, write_addr, write_data, read_data1,read_data2);

input clk;

input rst_n;

input [4:0] read_addr1;

input [4:0] read_addr2;

input write_en;

input [4:0] write_addr;

input [31:0] write_data;

output wire [31:0] read_data1;

output wire [31:0] read_data2;

reg [31:0] reg_mem [31:0];

initial begin

$readmemh("reg_memory.mem", reg_mem); //Load initial values

end

assign read_data1 = reg_mem[read_addr1];

assign read_data2 = reg_mem[read_addr2];

always @ (posedge clk or negedge rst_n)

begin

if (!rst_n)

begin

reg_mem[write_addr] <= reg_mem[write_addr];

end

else

begin

reg_mem[write_addr] <= write_en ? write_data : reg_mem[write_addr];

end

end

endmodule

- Register Memory (reg_memory.mem)

This is the register memory where the memory location corresponds to the particular register address.

We are using the following registers $t1, $t2, $t3 and $t4 in our code which corresponds to the register addresses 09, 10, 11 and 12.

Let us write some initial data 11 in $t1 and 22 in $t2. (locations 09 and 10)

Then the reg_memory.mem will look like this:

- ALU design in Verilog using MIPS Instruction Set

MIPS is a RISC (Reduced Instruction Set Computer) based architecture which is used in MIPS based processors.

Designing a simple ALU in Verilog using few example instructions from the MIPS Instruction Set.

MIPS Instructions are classified into three types: R, I and J. They have the following specified formats:

R-Type:

op - Opcode
rs, rt - Source Registers 1 and 2
rd - Destination Register
shamt - Shift Amount
funct - Function Code

I-Type:

J-Type:

Step 1: Decide the various instructions that ALU must support.

From the MIPS Instruction Set, ALU supports the following instructions:
AND, OR, ADD, SUB, SLT, NOR, LW, SW, BEQ.

Step 2: ALU Control Signal Generation

- For I-Type Instructions, the type of operation to be performed is found by looking at the opcode.

- For R-Type Instructions, the type of operation is determined by the function field (opcode field will remain as all zeroes).

Control signal for the ALU is required to determines the type of operation to be performed by looking at either the opcode for I-Type or function field for R-Type instructions.

In this case, it is as simple as: if the opcode field is all zeroes, then look at the function field.

- In LW and SW, perform addition as the offset must be added with the base address.

- ALU Control is the signal which is sent to the ALU core to indicate what type of operation is to be performed.

- For BEQ, subtraction must be performed. If the operands are equal, then the subtraction result will be 0 and the branching condition will be true. ALU will have a separate zero output signal to indicate output zero condition.

Verilog Module: ALU_Controller
Inputs: opcode (6 bits), func_field (6 bits)
Outputs: alu_control (3 bits)

Verilog Code for ALU Control:

module Alu_Control(opcode, func_field, alu_control);

input [5:0] opcode;

input [5:0] func_field;

output reg [2:0] alu_control;

reg [2:0] func_code; //A

always @ (*)

begin //for R type instructions

case (func_field)

6'h20: func_code = 3'h0;

6'h22: func_code = 3'h1;

6'h24: func_code = 3'h2;

6'h25: func_code = 3'h3;

6'h27: func_code = 3'h4;

6'h2A: func_code = 3'h5;

default: func_code = 3'h0;

endcase

case (opcode)

6'h00: alu_control = func_code;

6'h04: alu_control = 3'h1;

6'h23: alu_control = 3'h0;

6'h2B: alu_control = 3'h0;

default: alu_control = 3'h0;

endcase

end

endmodule

Step 3: ALU Core Design

Now that the control signal tells us the type of operation to be performed, the desired operation can be performed in the ALU core module. Let the input operands be A and B. The computed output is sent out as result.

Verilog module: ALU_Core
Inputs: alu_control (3 bits), A (32 bits), B (32 bits)
Outputs: result (32 bits), zero (1 bit)

Verilog Code for ALU Core:

module Alu_Core(A, B, alu_control, result, zero);

input [31:0] A;

input [31:0] B;

input [2:0] alu_control;

output reg [31:0] result;

output wire zero;

assign zero = !(|result); //oring the entire result to 1bit value then do negation

always @ (*)

begin

case(alu_control)

3'h0: result = A + B;

3'h1: result = A - B;

3'h2: result = A & B;

3'h3: result = A | B;

3'h4: result = ~(A | B);

3'h5: result = (A < B);

default: result = A + B;

endcase

end

endmodule

Step 4: Create the ALU top module.

The top module instantiates and connects both the above modules.

Verilog Module: ALU_Top
Inputs: opcode (6 bits), func_field (6 bits), A (32 bits), B (32 bits)
Outputs:  result (32 bits), zero (1 bit)

Verilog Code for ALU Top:

module Alu_Top(	opcode,	func_field, A, B, result, zero);

input [5:0] opcode;

input [5:0] func_field;

input [31:0] A;

input [31:0] B;

output [31:0] result;

output zero;

wire [2:0] alu_control;

Alu_Control alu_ctrlr_inst (.opcode (opcode),.func_field (func_field),.alu_control (alu_control));

Alu_Core alu_core_inst (.A (A),.B (B),.alu_control (alu_control),.result result),.zero (zero));

endmodule

The above Verilog code implements an ALU using certain instructions from the MIPS Instruction set.

- Sign Extension Unit:

The format of the lw and sw instructions is as follows.

- lw $t1, $t2, offset

- sw $t1, $t2, offset
where the offset_value is a signed 16-bit value.

Since we are working with 32-bit values, we will need to sign extend the 16-bit offset value to bring it to 32-bits, and so we require a Sign Extension Unit.

Verilog Code:

module Sign_Extension(

bits16_in,

bits32_out

);

input [15:0] bits16_in;

output wire [31:0] bits32_out;

assign bits32_out = {{16{bits16_in[15]}} , bits16_in[15:0]};

endmodule

- Data Memory:

- For lw and sw instructions, we are computing a data memory address from which we have to either fetch the data or store the data.

- lw $t1, $t2, offset

means-Fetch the data from this data memory address (value present in $t1 + sign-extended offset value) and store it in $t2

- sw $t1, $t2, offset 
means - The data present in $t2 has to be stored in the calculated data memory address  (value present in $t1 + sign-extended offset value)

Below is the Verilog code, again it is like the other memories we have designed above.We load data memory using readmemh function.

Verilog Code:

module Data_Memory(clk,address,write_en,write_data,read_data);

input clk;

input [31:0] address;

input write_en;

input [31:0] write_data;

output wire [31:0] read_data;

//Registers are addressed as per MIPS register table

reg [7:0] data_mem [31:0];

initial begin

$readmemh("data_memory.mem", data_mem);

end

assign read_data = {data_mem[address+3],data_mem[address+2],

data_mem[address+1],data_mem[address]};

always @ (posedge clk)

begin

data_mem[address]   <= write_en ? write_data[7:0]   : data_mem[address];

data_mem[address+1] <= write_en ? write_data[15:8]  : data_mem[address+1];

data_mem[address+2] <= write_en ? write_data[23:16] : data_mem[address+2];

data_mem[address+3] <= write_en ? write_data[31:24] : data_mem[address+3];

end

endmodule

- Shifter (for BEQ)

The final remaining instruction is the Branch Equal (BEQ) instruction.

Format: beq $t1, $t2, offset

Operation:

- Compare the two register values $t1 and $t2 to check for equality (can be done by subtraction operation in ALU and check for zero)

- If condition is false, we directly execute the next instruction (no issues here)

- If condition is true, we branch to the instruction with address equal to next instruction address(PC +4) + 18-bit offset (16-bit offset shifted left by 2 bits). To enable this shifting, we require a shifter module as well. For above address addition, we will require another small ALU.

- We do the shift left by 2 bits because for every offset value, we must increment address by 4 to reach the next address.
Example: Offset 1 means 1<< 2 = 4 meaning take the next instruction from address = address+4

Verilog Code:

module Shifter(

indata,

shift_amt,

shift_left,

outdata

);

input [31:0] indata;

input [1:0] shift_amt;

input shift_left;

output wire [31:0] outdata;

assign outdata = shift_left ? indata<<shift_amt : indata>>shift_amt;

endmodule

The above modules will be sufficient to design the datapath of the desired processor.

Now the job is to generate the control logic and create the top m

Overall Schematic

Working of the Processor

Starting with the Program Counter

1. At the first clock cycle, the Program Counter will be pointing to the address of the first instruction to be fetched.

2. This address goes to the Instruction Memory where the instruction will be fetched.

3. The instruction will consist of different fields depending on the MIPS instruction format as shown below:

Instruction [31:26] ⟶ opcode (To decide the type of instruction)

Instruction [25:21] ⟶ rs (address of register 1 to be read)

Instruction [20:16] ⟶ rt (address of register 2 to be read for R-Type instrn) or
                                        (address of register to be written with result for I-Type instrn)

Instruction [15:0] ⟶ const (offset value for I-Type Instruction)

For R-Type, Instruction [15:0] can be further split as:

Instruction [15:11] ⟶ rd (address of register to be written with result)

Instruction [10:6] ⟶ shamt (shift amount, not required for our case)

Instruction [5:0] ⟶ funct (function code)

4. All register operations are handled by the Register file. Hence, appropriate connections are made to it. Register file consists of the Register memory which addresses the inbuilt MIPS registers as we saw in the previous post.

5. The output data from Register file will go to any of the different modules depending on the type of instruction and is controlled by the control unit which we will discuss at the end.

For R-Type Instructions, the output from Register file goes to the ALU for performing the required computation and is written back to the Register file.

For LW and SW, it goes to the Sign Extension unit to extend the offset to 32-bits and then access the Data memory.

For BEQ, it goes to the shifter module where data is shifted left by 2 bits to convert the offset value into a processor address (aligned to 4-bytes).

Top Module

Let us look at the top module of the processor in Verilog. Here, we can see all the instances of each module and how they are connected as per the schematic.

Verilog Code:

module Processor_Top(clk,rst_n);

input clk;

input rst_n;

wire [31:0] ctrl_in_address;

wire [31:0] out_address;

wire [31:0] addr_incr;

wire [31:0] address_plus_4;

wire [31:0] branch_addr_offset;

wire [31:0] branch_address;

wire [31:0] instrn;

wire ctrl_write_en;

wire final_write_en;

wire [4:0] ctrl_write_addr;

wire [31:0] ctrl_regwrite_data;

wire [31:0] read_data1;

wire [31:0] read_data2;

wire [31:0] sign_ext_out;

wire [31:0] ctrl_aluin2;

wire [31:0] alu_result;

wire zero_out;

wire [31:0] datamem_read_data;

assign addr_incr = (!rst_n) ? 32'd0 : 32'd4;

assign final_write_en = (!rst_n) ? 1'b0 : ctrl_write_en;

Program_Counter prg_cntr (

.clk (clk),

.rst_n (rst_n),

.in_address (ctrl_in_address),

.out_address (out_address)

);

Adder adder_next_addr (

.in1 (out_address),

.in2 (addr_incr),

.out (address_plus_4)

);

Adder adder_branch_addr (

.in1 (address_plus_4),

.in2 (branch_addr_offset),

.out (branch_address)

);

Instruction_Memory instr_mem (

.instrn_address (out_address),

.instrn (instrn[31:0])

);

Register_File regfile (

.clk (clk),

.rst_n (rst_n),

.read_addr1 (instrn[25:21]),

.read_addr2 (instrn[20:16]),

.write_en   (final_write_en),

.write_addr (ctrl_write_addr),

.write_data (ctrl_regwrite_data),

.read_data1 (read_data1),

.read_data2 (read_data2)

);

Sign_Extension sign_ext (

.bits16_in (instrn[15:0]),

.bits32_out (sign_ext_out)

);

Shifter shifter (

.indata (sign_ext_out),

.shift_amt (2'd2),

.shift_left (1'b1),

.outdata (branch_addr_offset)

);

Alu_Top alu (

.opcode (instrn[31:26]),

.func_field (instrn[5:0]),

.A (read_data1),

.B (ctrl_aluin2),

.result (alu_result),

.zero (zero_out)

);

Data_Memory data_mem (

.clk		(clk),

.address (alu_result),

.write_en (ctrl_datamem_write_en),

.write_data (read_data2),

.read_data (datamem_read_data)

);

Control_Logic ctrl_logic (

.instrn			  (instrn),

.instrn_opcode   (instrn[31:26]),

.address_plus_4  (address_plus_4),

.branch_address  (branch_address),

.ctrl_in_address (ctrl_in_address),

.alu_result      (alu_result),

.zero_out        (zero_out),

.ctrl_write_en   (ctrl_write_en),

.ctrl_write_addr (ctrl_write_addr),

.read_data2      (read_data2),

.sign_ext_out    (sign_ext_out),

.ctrl_aluin2     (ctrl_aluin2),

.ctrl_datamem_write_en (ctrl_datamem_write_en),

.datamem_read_data (datamem_read_data),

.ctrl_regwrite_data (ctrl_regwrite_data)

);

endmodule

The Control Unit (Control Path)

- The control unit is that part of the processor which generates control signals in order to operate in the desired way depending on the instruction.

- For example: The next instruction addresses to be executed is PC+4 for normal cases but is calculated differently if it is a BEQ instruction.

Verilog Code:

module Control_Logic(

instrn,

instrn_opcode,

address_plus_4,

branch_address,

ctrl_in_address,

alu_result,

zero_out,

ctrl_write_en,

ctrl_write_addr,

read_data2,

sign_ext_out,

ctrl_aluin2,

ctrl_datamem_write_en,

datamem_read_data,

ctrl_regwrite_data

);

input [31:0] instrn;

input [5:0] instrn_opcode;

input [31:0] address_plus_4;

input [31:0] branch_address;

input [31:0] datamem_read_data;

input [31:0] alu_result;

input zero_out;

input [31:0] read_data2;

input [31:0] sign_ext_out;

output wire [31:0] ctrl_in_address;

output wire ctrl_write_en;

output wire [4:0] ctrl_write_addr;

output wire [31:0] ctrl_aluin2;

output wire ctrl_datamem_write_en;

output wire [31:0] ctrl_regwrite_data;

//Select either branch address (for BEQ) or address+4 (for other cases)

assign ctrl_in_address = (instrn_opcode==6'h04 && zero_out) ? branch_address : address_plus_4;

//Write enable for regfile only for R-Type and LW instructions

assign ctrl_write_en = (instrn_opcode==6'h00) || (instrn_opcode==6'h23);

//Write address for regfile selection based on R-Type and I-Type instructions

assign ctrl_write_addr = (instrn_opcode==6'h00) ? instrn[15:11] : instrn[20:16];

//Write data for regfile is from Data mem (LW) or from ALU

assign ctrl_regwrite_data = (instrn_opcode==6'h23) ? datamem_read_data : alu_result;

//Second input of ALU is either from regfile (R_Type) or sign ext unit (LW & SW)

assign ctrl_aluin2 = (instrn_opcode==6'h23 || instrn_opcode==6'h2B) ? sign_ext_out : read_data2;

//Write enable to datamem only for LW instruction

assign ctrl_datamem_write_en = (instrn_opcode==6'h2B);

endmodule

Result:

Simulation waveform

Analysis and Conclusion:

The first instruction with instruction address 00h which occurs at 30ns once the reset is pulled high.

The first instruction is add $t1, $t2, $t3

Operation to be done: Add the contents of $t1 and $t2 and place the result in $t3.

in_address is ready with current address 00h + 4.
out_address shows the present address 00h.

instrn is ready with the first instruction to be executed.

read_addr1 and read_addr2 are holding the addresses of the desired registers $t1 and $t2.

read_data1 and read_data2 are holding the data present in the above registers respectively.

These are sent as inputs to the ALU where addition is performed and the result is present in result.

write_addr holds the address of the register to be written, which is that of $t3 (0b in hex).

write_en is 1 meaning data is being written.

write_data holds the desired sum which is 0033h.